עידו כץ

אמילי מיאסקובסקי

1) Stakeholders

Primary End Users

- Hobbyist / Lab Operator: wants real-time alerts and a clear dashboard during printing.

- Student / Researcher: wants stored runs, statistics, and data export for analysis.

Secondary Stakeholders

- Project Team: develops, trains, deploys, and maintains the system.

- Supervisor / Evaluator: requires traceable requirements and measurable success criteria.

2) System Scope (What the System Does)

A Raspberry Pi captures images/video of an ongoing 3D printing process. An artificial intelligence model analyzes the stream in real time to detect failures (such as bed detachment, under-extrusion / lack of material, distortions, or unexpected stops). A web-based dashboard presents quality status and generates smart alerts.

3) Functional Requirements (FR)

FR-1 Live Print Session Management

- The system shall allow the user to start and stop a monitoring session.

- The system shall display current session metadata: printer name (optional), filament type (optional), start time, and current status.
Priority: Must
Effort: Medium (M)

FR-2 Real-Time Image/Video Capture

- The system shall capture frames from the camera at a configurable rate (e.g., 1–10 FPS).

- The system shall validate camera availability and notify the user if the camera fails.
Priority: Must
Effort: Medium (M)

FR-3 Real-Time Defect Detection (AI Core)

- The system shall run inference on incoming frames and output:

  - defect type (e.g., detachment, lack of material / under-extrusion, distortion / “spaghetti” / stringing, unexpected stop)

  - confidence score

  - timestamp

- The system shall support a “No defect detected” state.
Priority: Must
Effort: Large (L)

FR-4 Smart Alerts and Notifications

- The system shall trigger alerts when the confidence score exceeds a defined threshold.

- The system shall support at least:

  - dashboard alert (visual)

  - sound alert (optional)

  - email / Telegram / WhatsApp / phone call (optional – stretch goal)

- The user shall be able to configure alert thresholds per defect type.
Priority: Must (dashboard alerts), Should (external notifications)
Effort: Medium–Large (M–L)

FR-5 Dashboard – Live Monitoring View

- The dashboard shall display:

  - live camera feed (or near real-time snapshots)

  - current status (OK / Warning / Failure)

  - latest detected defect and confidence score

  - timeline of detections and events
Priority: Must
Effort: Medium (M)

FR-6 Run History and Reports

- The system shall store completed sessions including:

  - session timestamps

  - detection events

  - summary metrics (e.g., number of alerts, alert times, print date)

  - printed model used in the session

- The dashboard shall provide a history page to view and filter past runs.
Priority: Should
Effort: Medium (M)

FR-7 Evidence Capture (Snapshots on Events)

- For each alert, the system shall store the frame that triggered the alert and display it on the dashboard.
Priority: Should
Effort: Medium (M)

FR-8 User Management (Optional)

- The system may support multiple users with login functionality.
Effort: Medium–Large (M–L)

4) Non-Functional Requirements (NFR)

NFR-1 Real-Time Performance

- The system shall display detection results within ≤ 2 seconds from frame capture (target).

- The dashboard shall update at least once every 1–2 seconds during an active session.

NFR-2 Accuracy Targets (Project KPI)

- The model should achieve (example targets):

  - Precision ≥ 0.85 – out of all cases where the model predicts a defect, how many are true defects?

  - Recall ≥ 0.80 – out of all real defects that occurred, how many were successfully detected?

- The system shall log false positives and false negatives for evaluation.

NFR-3 Reliability

- Monitoring service uptime during a session: ≥ 99% (no crashes during normal printing).

- If inference fails, the system shall log the error.

NFR-4 Usability

- The dashboard must be readable on a laptop.

- Alerts must be clear and must not require technical expertise to interpret.

NFR-5 Security and Privacy

- Dashboard access shall be protected (at minimum by password authentication).

- Video/images are stored locally by default; remote access is optional and must be secure.

5) Use Cases

UC-1 Start Print Monitoring

Actors: User
Preconditions: Raspberry Pi online; camera connected; dashboard reachable.
Postconditions: Session created; live stream and inference running; dashboard shows “Monitoring Active”.

Basic Flow:

- User opens the dashboard.

- User uploads the model and clicks “Start Session”.

- System checks camera and inference availability.

- System starts capturing frames and running detection.

- Dashboard displays live feed and “OK” status.

Alternate Flows:

- A1: Camera not found → system shows error and troubleshooting tips; session not started.

- A2: Model not loaded → system shows error and offers “Restart AI Service”.

UC-2 Detect Failure and Alert User

Actors: System, User
Preconditions: UC-1 active session running.
Postconditions: Alert created; event stored; user notified.

Basic Flow:

- System captures a frame.

- AI model detects a defect with confidence above threshold.

- System creates an alert event and saves a snapshot.

- Dashboard shows alert banner with defect details and confidence.

- (Optional) External notification is sent.

Alternate Flows:

- A1: Low-confidence detection → logged as warning without alert.

- A2: Storage full → monitoring continues but snapshots are not saved; warning shown.

UC-3 Review Past Session Results

Actors: User
Preconditions: At least one session exists.
Postconditions: User views session summary and events.

Basic Flow:

- User opens “History”.

- User selects a session.

- System displays timeline, alerts, snapshots, and summary metrics.

Alternate Flows:

- A1: Session corrupted or missing → system shows “Cannot load” and logs error.

6) User Stories and Acceptance Criteria

US-1 Live Status

As a print operator, I want to see the current print status (OK / Warning / Failure) so that I can react quickly.

Acceptance Criteria:

- Status updates at least every 2 seconds.

- Status changes are logged with timestamps.

US-2 Real-Time Alerts

As a user, I want to receive an alert when the print detaches or fails so that I can stop the print and avoid waste.

Acceptance Criteria:

- When confidence ≥ threshold, an alert is shown on the dashboard within 2 seconds.

- Alert includes defect type, confidence, and snapshot.

US-3 Sensitivity Configuration

As a user, I want to adjust detection thresholds to reduce false alarms.

Acceptance Criteria:

- Threshold can be configured per defect type.

- New thresholds apply immediately to the current session.

US-4 History and Evidence

As a student/researcher, I want to review past sessions with snapshots to evaluate model performance.

Acceptance Criteria:

- Each alert includes a stored image.

- Sessions can be filtered by date and defect type.

7) Prioritization and Effort Estimation (MoSCoW)

- FR-1 Session start/stop and metadata – Must – M

- FR-2 Frame capture – Must – M

- FR-3 Real-time defect detection – Must – L

- FR-4 Alerts and thresholds – Must – M–L

- FR-5 Live dashboard view – Must – M

- FR-6 History and reports – Should – M

- FR-7 Snapshot on alert – Should – M

- FR-8 Multi-user roles – Could – M–L

8) Technological Requirements (TR)

TR-1 Hardware

- Raspberry Pi (recommended: Pi 4 / Pi 5) with:

  - camera module (official or USB)

  - stable power supply

  - local storage (microSD; optional external SSD)

TR-2 Operating System and Runtime

- Raspberry Pi OS (Linux)

- Python environment for capture and inference

TR-4 Streaming and Backend Services

- Local backend service exposing APIs:

  - REST for session control and history

  - WebSocket (or similar) for real-time updates

TR-5 Frontend – Dashboard

- Web-based dashboard accessible via browser on the same network.

- Responsive interface (desktop).

TR-6 Data Storage

- Local database/file storage:

  - event log (SQLite or JSONL)

  - snapshots with timestamps

TR-7 Security

- Dashboard protected by basic authentication.

- For remote access: HTTPS (or VPN) or SSH.

TR-8 Monitoring, Logging, and Debugging

- Logs for camera status, inference errors, and alert events.

- Export of logs and session data for evaluation.

9) High-Level Architecture Requirements

Components

- Capture Service (Pi): camera acquisition → frame queue

- Inference Service (Pi): preprocessing → inference → event generation

- Storage (Pi): event database and snapshots

- Backend API (Pi or PC): session control and data endpoints

- Web Dashboard (Browser): live view, alerts, and history

Communication

- Capture → Inference: in-memory queue

- Inference → Backend/Dashboard: WebSocket for events and REST for history